# SOHO Network Setup - Questions and Answers

## Task

1. Draw a network diagram for a SOHO setup with ISP, router, laptops, smart TV, printer.

## Answer

## Task

2. Hardware components required for SOHO network.

## Answer

Hardware List:
- Wi-Fi Router: TP-Link Archer C6 / Netgear Nighthawk
- Broadband Modem: ISP provided (Jio/Airtel/BSNL)
- Laptops: HP/Dell/Lenovo
- Smart TV: Samsung/LG Smart TV
- Printer: HP WiFi Printer
- Ethernet Cable: Cat6 RJ45
- Optional: 5-port Gigabit Switch, Wi-Fi extender

## Task

3. Steps to configure Wi-Fi router.

## Answer

Steps:
1. Connect modem to router WAN port.
2. Open 192.168.0.1 in browser.
3. Set SSID name (SOHO_Network).
4. Enable WPA2/WPA3 security.
5. Change admin username/password.
6. Enable DHCP for automatic IP assignment.
7. Save and reboot router.

## Task

4. IP configuration table for SOHO network.

## Answer

Device        MAC Address            IP Address
Laptop1       AA:11:22:33:44:55      DHCP
Laptop2       BB:22:33:44:55:66      DHCP
Smart TV      CC:33:44:55:66:77      192.168.1.50
Printer       DD:44:55:66:77:88      192.168.1.60

Subnet Mask: 255.255.255.0
Gateway: 192.168.1.1

## Task

5. Troubleshooting laptop internet issue.

## Answer

Possible Causes:
1. Wrong IP configuration → Renew DHCP / restart adapter
2. Weak WiFi signal → move closer or use extender
3. DNS/router issue → restart router or change DNS

Steps:
- ping router IP
- check WiFi status
- run ipconfig /renew